# WiFi Authentication Matrix Test Suite

# Introduction

## Motivation

All communication requires two parties. For WiFi, the two parties are Access Points (referred to as APs) and Mobile Devices (referred to as STA or “Stations”).  In order to communicate securely, the two parties, the APs and STAs, must be able to trust and understand each other. These two requirements are fulfilled by processes known as Authentication and Encryption. The former enables the parties to establish their identities with a fair degree of certainty and the latter allows them to exchange messages securely.

A detailed description of the available authentication and encryption techniques is beyond the scope of this document. Instead, we just list the primary protocols available in January, 2006:

Authentication Methods:

Open: all associations accepted

Shared: all associations accepted but client must use WEP encryption

WPA: WiFi Protected Access (requires EAP authentication)

WPA-PSK: WPA with a Private Shared Key

WPA2: WiFi Protected Access 2 (requires EAP authentication)

WPA2-PSK: WPA2 with a Private Shared Key

Encryption Methods:

ClearText: messages not encrypted

WEP: Wired Equivalent Privacy

TKIP: Temporal Key Integrity Protocol

AES: Advanced Encryption Standard

EAP (Extensible Authentication Protocol) Authentication Methods

TLS: Transport Layer Security

MD5: Message Digest 5

PEAP: Protected Extensible Authentication Protocol

Validating proper operation of a WiFi STA requires testing its ability to connect using all the valid combinations of the preceding authentication and encryption protocols. Additionally, it requires “negative” testing to make sure the STA will not connect when the Access Point is configured with a different set.

Performing all these tests manually is a time-consuming and error-prone challenge. The Authentication-Matrix test suite automates the process. For each combination, the test suite:

- Connects with a configuration server using a fixed-configuration AP,

- Selects an AP to be configured and adjusts it to the appropriate authentication, encryption and key,

- Disconnects from the fixed-configuration AP,

- Configures the STA under test to the authentication, encryption and key we’re testing,

- Waits a fixed period for the connection to be established,

- Sends a large number of ICMP pings through the newly-connected WiFi link,

- Sends a large number of UDP echoes and checks for lost or corrupted replies,

- Sends a large number of TCP echoes and checks for lost or corrupted replies, and

- Disconnects the wireless adapter.

## Methodology

The Authentication Matrix test suite is implemented as a tux dll. As such, it can be started and configured remotely using CETK or CE Platform Builder or locally using a command-script.

In addition to the Access Points and the device under test, the suite requires a Radius server for EAP authentication, a DHCP server for IP address assignments and an Access Point Control server to configure the APs:

# Configuration

## Network and Hardware Setup

The Access Point control server is a Windows application which fields AP-configuration requests from the devices under test and updates the AP to the required security modes. Additionally, the same computer must be running both UDP and TCP Echo services so the devices under test can verify wireless connections. (These services are usually provided by the “Simple TCP/IP Services” Windows component.)

The AP-control, Radius and DHCP servers must share a common subnet so they can be contacted by the devices under test. This subnet should be isolated from systems containing sensitive information because a few of the authentication modes (particularly Open with ClearText) provide little, if any, protection against hacking.

The diagram shows an optional programmable RF Attenuation device. When equipped with an RF Attenuator, the AP-control server can dynamically adjust the strength of the RF signal received by the devices being tested. Although this capability is not currently utilized, it may be in the future.

## Radius Server Configuration

All the Access Points used for the test must be in the Radius Server’s list of clients. Select and enter a passphrase for each. These keys must be configured into the corresponding Access Points so they can authenticate themselves with the Radius server during EAPOL key processing. (See Access Point Configuration below.)

If, as is usually the case, your Radius server is also your Authentication server, it needs two special user accounts; one for TLS EAP authentication and one for PEAP. The tests default to the following credentials for these accounts:

TLS authentication:

User-name: eaptls

Password: eaptls

Domain: wince

PEAP authentication:

User-name: eappeap

Password: eappeap

Domain: wince

If these defaults are not appropriate, see Test Configuration below about modifying the test configuration accordingly.

## Access Point Configuration

### Supported Access Points:

These Access Point types are currently supported by the AP-Control server:

| Vendor/Model | Firmware | Supported Modes |
| --- | --- | --- |
| Buffalo AirStation WBR2-G54S | Version 2.30 or higher | Open Shared WPA WPA-AES WPA-PSK |
| Buffalo AirStation WBR2-G54S | Broadcom: CFE 3.52.21.10 Linux 3.131.20.0 | Open Shared WEP-802.1X WPA WPA-AES WPA-PSK WPA2 WPA2-PSK |
| D-Link DL-524 | ToDo | Open Shared WPA WPA-PSK |
| D-Link DWL-3200 | Version 2.10 or higher | Open Shared WPA WPA-AES WPA-PSK WPA2 WPA2-PSK |

### Fixed Access Point:

Set up one of the APs to be the “fixed” association point for devices connecting to the AP-control server:

The authentication mode for the Fixed AP must not require 802.1X (EAP) authentication. Currently, that means it must be one of Open, Shared, WPA-PSK or WPA2-PSK.

This is because, at the beginning of the test, the devices will not have the certificates required to authenticate themselves with the Radius server. During the first test in the suite the application retrieves the certificates but it must communicate with the Authentication server to do so:

This certificate-retrieval procedure has a few other subtle aspects:

- The certificate retrieval is done after the first successful wireless connection. If the –lHost option is used to connect the wireless device and server over a LAN, the retrieveal will not be done until the end of the first successful wireless-connection test.

- There are numerous options for selecting the tests to be performed. If the selected tests contain nothing but EAP, these will all fail due to the lack of a preceding wireless connection and certificate retrieval.

### Controllable Access Points:

Manually configure the remaining controllable Access Points so they communicate properly with your Radius server. This requires configuring the server’s IP-address and port (usually 1812) and the passphrase you selected for the AP. (See Radius Server Configuration above.)

Configure all the APs with static IP addresses on the test sub-net. Connect to each of the APs from the AP-Control system to test that connection and retrieve the APs’ initial, fixed, configuration information:

- SSID

- IP address

- Administrator user-name and password

## AP-Control Server Configuration

The AP-Control server is an application running on an XP or Windows 2003 system attached to the test subnet. On startup it registers itself, reads its configuration and awaits requests to update the configurations of the Access Points under its control.

### Command-Line Arguments:

APControl [-s ServerHost] [-p Port] [-k Registry-Key]

options:

-k   registry key (default: Software\Microsoft\CETest\APCTL)

-s   name/address of server (default: localhost)

-p   port to connect on (default: 33331)

miscellaneous:

-z   send debug output to console

-v   verbose debug output

-?   show this information

Normally, the only options supplied are “-p” and “-k” for specifying the port number and registry key. The former tells the server which TCP/IP port it should use for accepting connections. The latter tells the server where to read the initial configurations of the Access Points it is to control.

### Access Point Registry Settings:

An example registry file describing two Access Points looks like this:

Windows Registry Editor Version 5.00

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP1]

"CapsImplemented"="WPA,WPA_PSK,WPA2,WPA2_PSK"

"CapsEnabled"="WPA,WPA_PSK,WPA2,WPA2_PSK"

"RadioState"=dword:00000001

"SSID"="CE_DLink_WPA2_AuthMatrix1"

"BSSID"=hex:00,13,46,89,b9,db

"Authentication"="WPA2_PSK"

"Cipher"="TKIP"

"RadiusServer"="10.10.0.1"

"RadiusPort"=dword:00001812

"RadiusSecret"="0123456789"

"WEPIndex"=dword:00000000

"WEPKey0"=hex:a0,b0,c0,d0,e0

"WEPKey1"=hex:f1,a2,b3,c4,d5

"WEPKey2"=hex:e2,f3,a4,b5,c6

"WEPKey3"=hex:d3,e4,f5,a6,b7

"Passphrase"="0123456789"

"Configurator"="dlink;10.10.0.48"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP2]

"CapsImplemented"="WEP_802_1X,WPA,WPA_PSK,WPA2,WPA2_PSK"

"CapsEnabled"="WEP_802_1X,WPA,WPA_PSK,WPA2,WPA2_PSK"

"RadioState"=dword:00000001

"SSID"="CE_Buffalo_WPA2_AuthMatrix"

"BSSID"=hex:00,13,46,89,b9,db

"Authentication"="WPA2_PSK"

"Cipher"="TKIP"

"RadiusServer"="10.10.0.1"

"RadiusPort"=dword:00001812

"RadiusSecret"="0123456789"

"WEPIndex"=dword:00000000

"WEPKey0"=hex:a0,b0,c0,d0,e0

"WEPKey1"=hex:f1,a2,b3,c4,d5

"WEPKey2"=hex:e2,f3,a4,b5,c6

"WEPKey3"=hex:d3,e4,f5,a6,b7

"Passphrase"="0123456789"

"Configurator"="buffalo;10.10.0.16:admin:admin"

"Attenuator"="Manual;0,0,100"

These registry settings describe two Access Points. The first, named “AP1”, is a D-Link AP with SSID “CE_DLink_WPA2_AuthMatrix” which can be contacted at IP address 10.10.0.48 with the default D-Link administrator name and password. The second, named “AP2”, is a Buffalo AP with SSID “CE_Buffalo_WPA2_AuthMatrix” which can be contacted at IP address 10.10.0.16 with administrator name and password “admin” and “admin”, respectively,

Most of these settings are optional and, if they are supplied by the user, ignored. The server retrieves them from the Access Point itself.

The required settings are SSID, Attenuator, and Configurator:

#### SSID:

This setting contains the SSID of the Access Point.

#### Attenuator:

Format: {device-type};{current},{minimum},{maximum}

The current, minimum and maximum values are dBs of attenuation.

Although this capability is not used by the Authentication Matrix test suite, the AP-Control server is programmed to control an RF Attenuator for adjusting the RF output strength of each Access Point. This setting tells the server how to contact and control the attenuator. Since the capability is not used, this should always be set to “Manual;0,0,100”.

#### Configurator:

Format: {device-type};{name|address}[:{admin-user}:{admin-password}]

This setting tells the server how to contact the Access Point. It has two components: the basic AP type and its address. Currently, the AP type should be either “dlink”, “buffalo” or “manual”.

The AP address usually contains the IP address of the Access Point. If the administrator name or password has been changed from the factory defaults, the new name and password are placed immediately following the address.

The “manual” AP driver pops up a dialog box on the AP-Control system console describing the new AP settings. When the dialog appears, the operator should manually update the AP’s configuration. The test allows approximately ten minutes for an “OK” response before the suite fails and move on to the next test.

## Test Configuration

The Authentication Matrix test suite is a Tux DLL running on the device under test. On startup it contacts the device’s wireless adapter, lists its current state and performs a few basic tests to make sure the adapter is operating normally. After that, the suite tests each of the security mode combinations as described earlier. (See the Appendix for a complete list of the tests.)

### Command-Line Arguments:

s tux -o -d AuthMatrix.dll -c [options]

-?  show this information

#### General Options:

-adapter   wireless adapter name (default first available)

If this option is not supplied, the tests will use the first wireless adapter they find. If the device under test contains multiple wireless adapters, use this option specify the adapter to be tested.

#### AP Control Server Options:

-lHost     LAN server name/address (default "157.56.97.94")

-lPort     LAN server port (default 33331)

Usually, the device to be tested has no wired LAN connection. If yours does, use these options to specify the name/address and port number where the AP-Control serve can be reached.

-wHost     WiFi server name/address (default "10.10.0.1")

-wPort     WiFi server port (default 33331)

-wSSID     WiFi server SSID (default "")

-wAuth     WiFi server authentication (default "Open")

-wEncr     WiFi server encryption-cipher (default "ClearText")

-wEap      WiFi server EAP auth-mode (default "TLS")

-wIndex    WiFi server WEP-key index (default 0)

-wKey      WiFi server encryption-key (default "")

If the SSID is specified, the LAN server will not be used. Instead, all server communication will use this Access Point. Note that the following options must be specified to associate with the Access Point:

-wAuth options:

Open

Shared

WPA_PSK

WPA2_PSK

-wEncr options:

ClearText

WEP

TKIP

AES

-wIndex options:

1 (or 0)

2

3

4

-wKey examples:

(WEP-40) 01.02.03.04.05

(WEP-104) 12.34.56.78.90.AB.CD.EF.12.34.56.78.90

(PSK) qwertyuiopasdfghjklzxcvbnm

(PSK) 0123456789

#### Certificate Enrollment Options:

-nHost     enrollment host name

(default "10.10.0.1")

-nCommand  enrollment command name

(default "enroll")

-nRootBox  title of "set root cert" dialog-box

(default "Root Certificate Store")

-nToolBox  title of "insert new cert" dialog-box

(default "Enrollment Tool")

-nWaitTime milliseconds to wait for command

(default 900000 (15 mins))

Unless the EAP tests are disabled (see below), the first test automatically retrieves certificates for authenticating EAP connections. These settings tell the application how to reach the Authentication server and how to run the certificate-enrollment program. The latter should usually be left as is.

#### User Logon Options:

-uUsrBox   title of user logon dialog-box

(default "User Logon")

-uNetBox   title of network password dialog-box

(default "Enter Network Password")

-uWaitTime milliseconds to wait for dialog close

(default 300000 (5 mins))

Each time the tests connect using WEP 802.1X, WPA or WPA2, they are required to authentication themselves with the Radius server. These settings tell the application how to interact with the device’s user-credentials dialogs. Usually the application is able to provide the credentials directly so the dialogs are not used. In any case, the defaults are usually correct so these options should be left as is.

#### Authentication Matrix Test Options:

-dOpen     disable (skip) Open System (no authentication) tests

-dShared   disable (skip) Shared (WEP authentication) tests

-d802      disable (skip) 802.1X (dynamic WEP) tests

-dEAP      disable (skip) EAP (Radius) tests

(WEP 802.1X, WPA and WPA2)

-dPSK      disable (skip) PSK tests (WPA-PSK and WPA2-PSK)

-dWPA2     disable (skip) WPA2 tests (AES, WPA2 and WPA2-PSK)

-dClear    disable (skip) ClearText (no encryption) tests

-dWEP      disable (skip) WEP encryption tests

-dTKIP     disable (skip) TKIP encryption tests

-dAES      disable (skip) AES encryption tests

-dPEAP     disable (skip) PEAP EAP-authentication tests

-dTLS      disable (skip) TLS EAP-authentication tests

These options allow you to skip individual segments of the tests. For example, specifying “-dOpen –dShared” tells the test suite to skip both the Open and Shared authentication tests.

-tAPNames  choose from comma-separated list of AP names

(default any)

If there are multiple Access Points to choose from, the test suite normally decides which to use in this order:

- If there is an AP with a matching security mode, use it.

- If not all the APs support the security modes, use the first that does.

- Use the first AP in the list.

The test normally selects amongst all the APs controlled by the AP-Control server. If this option is specified, the application will only select amongst the named APs.

-lTLS      TLS login credentials

(default "eaptls:eaptls:wince")

-lPEAP     PEAP login credentials

(default "eappeap:eappeap:wince")

These options specify the EAP login credentials. The argument format is {user-name}:{password}:{domain-name}.

-tPasses   number times to repeat each connection (default 1)

-tConnTime seconds to await a connection (default 140 seconds)

These options specify the number of times to repeat each test and the time to wait for each test to finish.

# Examples

## Single Server and Access Point

For these tests there are two Access Points (one fixed and one controllable), a single Windows server at IP address 10.10.0.1 running APControl, Radius and DHCP, a test network and a mobile device.

The fixed AP is:

SSID: CE_Cisco_WPA2_AuthMatrix

Authentication: Open

Encryption: WEP

WEP-Key Index: 1

WEP key: 0123456789

### AP-Control Server Configuration

Access Point registry entries:

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP1]

"SSID"="CE_DLink_WPA2_AuthMatrix1"

"Configurator"="dlink;10.10.0.48"

"Attenuator"="Manual;0,0,100"

Command-line:

apcontrol.exe –v –z –k Software\Microsoft\CETest\APCTL

### Mobile-Device Configuration

Test-suite Command-Line:

\Windows\tux.exe -f \temp\amdebug.log -d authmatrix.dll

-c "-wHost 10.10.0.1

-wSSID CE_Cisco_WPA2_AuthMatrix

-wAuth open -wEncr wep -wKey 0123456789

-tConnTime 300"

(Usually, this command-line is packaged into a CE shortcut file and installed along with the tux binaries and the test suite DLL “AuthMatrix.dll”.)

## Radius Server, AP-Control Server and Multiple Access Points

For these tests there are four Access Points (one fixed and three controllable), an AP-Control server at IP address 10.10.0.2, another server running Radius and DHCP at IP address 10.10.0.1, a test network and a mobile device.

The fixed AP is:

SSID: CE_Cisco_WPA2_AuthMatrix

Authentication: WPA-PSK

Encryption: TKIP

WEP key: qwertyuiop

### AP-Control Server Configuration

Access Point registry entries:

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP1]

"SSID"="CE_DLink_WPA_AuthMatrix"

"Configurator"="dlink;10.10.0.46"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP2]

"SSID"="CE_Broadcom_WPA_AuthMatrix"

"Configurator"="buffalo;10.10.0.47:admin:admin"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\AP3]

"SSID"="CE_DLink_WPA2_AuthMatrix"

"Configurator"="dlink;10.10.0.48:dlinkroot:dlinkpasswd"

"Attenuator"="Manual;0,0,100"

Command-line:

apcontrol.exe –v –z –k Software\Microsoft\CETest\APCTL

### Mobile-Device Configuration

Test-suite Command-Line:

\Windows\tux.exe -f \temp\amdebug.log -d authmatrix.dll

-c "-wHost 10.10.0.2 -wPort 33331

-wSSID CE_Cisco_WPA2_AuthMatrix

-wAuth wpa-psk -wEncr tkip -wKey qwertyuiop

-tConnTime 300"

## Multiple Concurrent Tests

For these tests there are five Access Points (one fixed and four controllable), a single Windows server at IP address 10.10.0.1 running APControl, Radius and DHCP, a test network and three mobile devices.

The fixed AP is:

SSID: CE_Cisco_WPA2_AuthMatrix

Authentication: Shared

Encryption: WEP

WEP-Key Index: 1

WEP key: 0123456789

### AP-Control Server Configuration

Access Point registry entries:

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\TEST1]

"SSID"="CE_DLink_WPA_AuthMatrix"

"Configurator"="dlink;10.10.0.46"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\TEST2_WPA2]

"SSID"="CE_DLink_WPA_AuthMatrix"

"Configurator"="dlink;10.10.0.46"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\TEST2_WEP_802]

"SSID"="CE_Broadcom_WPA_AuthMatrix"

"Configurator"="buffalo;10.10.0.47:admin:admin"

"Attenuator"="Manual;0,0,100"

[HKEY_CURRENT_USER\Software\Microsoft\CETest\APCTL\TEST3]

"SSID"="CE_DLink_WPA2_AuthMatrix"

"Configurator"="dlink;10.10.0.48:dlinkroot:dlinkpasswd"

"Attenuator"="Manual;0,0,100"

Command-line:

apcontrol.exe –v –z –k Software\Microsoft\CETest\APCTL

### Mobile-Device Configuration

Test-suite Command-Line – Mobile 1:

\Windows\tux.exe -f \temp\amdebug.log -d authmatrix.dll

-c "-wHost 10.10.0.1

-wSSID CE_Cisco_WPA2_AuthMatrix

-wAuth shared -wEncr wep -wKey 0123456789

-tAPNames TEST1

-tConnTime 300"

Test-suite Command-Line – Mobile 2:

\Windows\tux.exe -f \temp\amdebug.log -d authmatrix.dll

-c "-wHost 10.10.0.1

-wSSID CE_Cisco_WPA2_AuthMatrix

-wAuth shared -wEncr wep -wKey 0123456789

-tAPNames TEST2_WPA2,TEST2_WEP_802

-tConnTime 300"

Test-suite Command-Line – Mobile 3:

\Windows\tux.exe -f \temp\amdebug.log -d authmatrix.dll

-c "-wHost 10.10.0.1

-wSSID CE_Cisco_WPA2_AuthMatrix

-wAuth shared -wEncr wep -wKey 0123456789

-tAPNames TEST3

-tConnTime 300"

# Appendix

## Basic Wireless-access Tests:

1000: Display WZC wireless-adapter information

1001: Display WZC context information

1002: Disables the WZC service

1003: Enables the WZC service

1004: Clears the current WZC Preferred Networks List

## Authentication Matrix Tests:

2000: Auth=Open Cipher=ClearText

2100: Auth=Open Cipher=WEP 40-bit key (random)

2110: Auth=Open Cipher=WEP 40-bit key (semi-null)

2120: Auth=Open Cipher=WEP 40-bit key (semi-ones)

2130: Auth=Open Cipher=WEP 104-bit key (random)

2140: Auth=Open Cipher=WEP 104-bit key (semi-null)

2150: Auth=Open Cipher=WEP 104-bit key (semi-ones)

2200: (Negative test)AP: Auth=Open Cipher=WEP

STA: Auth=Open Cipher=TKIP

2300: (Negative test) AP: Auth=Open Cipher=WEP

STA: Auth=Open Cipher=AES

3000: (Negative test) AP: Auth=Shared Cipher=WEP

STA: Auth=Shared Cipher=ClearText

3100: Auth=Shared Cipher=WEP 40-bit key (random)

3110: Auth=Shared Cipher=WEP 40-bit key (semi-null)

3120: Auth=Shared Cipher=WEP 40-bit key (semi-ones)

3130: Auth=Shared Cipher=WEP 104-bit key (random)

3140: Auth=Shared Cipher=WEP 104-bit key (semi-null)

3150: Auth=Shared Cipher=WEP 104-bit key (semi-ones)

3200: (Negative test) AP: Auth=Shared Cipher=WEP

STA: Auth=Shared Cipher=TKIP

3300: (Negative test) AP: Auth=Shared Cipher=WEP

STA: Auth=Shared Cipher=AES

4000: (Negative test) AP: Auth=WEP_802_1X Cipher=WEP

STA: Auth=WEP_802_1X Cipher=ClearText

4100: Auth=WEP_802_1X Cipher=WEP EAP=TLS

4120: Auth=WEP_802_1X Cipher=WEP EAP=PEAP

4200: (Negative test) AP: Auth=WEP_802_1X Cipher=WEP

STA: Auth=WEP_802_1X Cipher=TKIP

4300: (Negative test) AP: Auth=WEP_802_1X Cipher=WEP

STA: Auth=WEP_802_1X Cipher=AES

5000: (Negative test) AP: Auth=WPA Cipher=TKIP

STA: Auth=WPA Cipher=ClearText

5100: (Negative test) AP: Auth=WPA Cipher=TKIP

STA: Auth=WPA Cipher=WEP

5200: Auth=WPA Cipher=TKIP EAP=TLS

5220: Auth=WPA Cipher=TKIP EAP=PEAP

5300: Auth=WPA Cipher=AES EAP=TLS

5320: Auth=WPA Cipher=AES EAP=PEAP

6000: (Negative test) AP: Auth=WPA_PSK Cipher=TKIP

STA: Auth=WPA_PSK Cipher=ClearText

6100: (Negative test) AP: Auth=WPA_PSK Cipher=TKIP

STA: Auth=WPA_PSK Cipher=WEP

6200: Auth=WPA_PSK Cipher=TKIP 63-digit passphrase (random)

6210: Auth=WPA_PSK Cipher=TKIP 8-digit passphrase (random)

6220: (Negative test) Auth=WPA_PSK Cipher=TKIP 7-digit passphrase

6230: Auth=WPA_PSK Cipher=TKIP 63-digit passphrase (semi-ones)

6240: Auth=WPA_PSK Cipher=TKIP 8-digit passphrase (semi-ones)

6250: (Negative test) Auth=WPA_PSK Cipher=TKIP 64-digit passphrase

6260: Auth=WPA_PSK Cipher=TKIP 63-digit passphrase (semi-null)

6270: Auth=WPA_PSK Cipher=TKIP 8-digit passphrase (semi-null)

6300: Auth=WPA_PSK Cipher=AES 63-digit passphrase (random)

6310: Auth=WPA_PSK Cipher=AES 8-digit passphrase (random)

6320: (Negative test) Auth=WPA_PSK Cipher=AES 7-digit passphrase

6330: Auth=WPA_PSK Cipher=AES 63-digit passphrase (semi-ones)

6340: Auth=WPA_PSK Cipher=AES 8-digit passphrase (semi-ones)

6350: (Negative test) Auth=WPA_PSK Cipher=AES 64-digit passphrase

6360: Auth=WPA_PSK Cipher=AES 63-digit passphrase (semi-null)

6370: Auth=WPA_PSK Cipher=AES 8-digit passphrase (semi-null)

7000: (Negative test) AP: Auth=WPA2 Cipher=TKIP

STA: Auth=WPA2 Cipher=ClearText

7100: (Negative test) AP: Auth=WPA2 Cipher=TKIP

STA: Auth=WPA2 Cipher=WEP

7200: Auth=WPA2 Cipher=TKIP EAP=TLS

7220: Auth=WPA2 Cipher=TKIP EAP=PEAP

7300: Auth=WPA2 Cipher=AES EAP=TLS

7320: Auth=WPA2 Cipher=AES EAP=PEAP

8000: (Negative test) AP: Auth=WPA2_PSK Cipher=TKIP

STA: Auth=WPA2_PSK Cipher=ClearText

8100: (Negative test) AP: Auth=WPA2_PSK Cipher=TKIP

STA: Auth=WPA2_PSK Cipher=WEP

8200: Auth=WPA2_PSK Cipher=TKIP 63-digit passphrase (random)

8210: Auth=WPA2_PSK Cipher=TKIP 8-digit passphrase (random)

8220: (Negative test) Auth=WPA2_PSK Cipher=TKIP 7-digit passphrase

8230: Auth=WPA2_PSK Cipher=TKIP 63-digit passphrase (semi-ones)

8240: Auth=WPA2_PSK Cipher=TKIP 8-digit passphrase (semi-ones)

8250: (Negative test) Auth=WPA2_PSK Cipher=TKIP 64-digit passphrase

8260: Auth=WPA2_PSK Cipher=TKIP 63-digit passphrase (semi-null)

8270: Auth=WPA2_PSK Cipher=TKIP 8-digit passphrase (semi-null)

8300: Auth=WPA2_PSK Cipher=AES 63-digit passphrase (random)

8310: Auth=WPA2_PSK Cipher=AES 8-digit passphrase (random)

8320: (Negative test) Auth=WPA2_PSK Cipher=AES 7-digit passphrase

8330: Auth=WPA2_PSK Cipher=AES 63-digit passphrase (semi-ones)

8340: Auth=WPA2_PSK Cipher=AES 8-digit passphrase (semi-ones)

8350: (Negative test) Auth=WPA2_PSK Cipher=AES 64-digit passphrase

8360: Auth=WPA2_PSK Cipher=AES 63-digit passphrase (semi-null)

8370: Auth=WPA2_PSK Cipher=AES 8-digit passphrase (semi-null)

Note that we are describing the WiFi networking scenario known as Infrastructure Mode. Another scenario, Adhoc Mode, describes temporary networks established between independent mobile devices. Adhoc networks are not tested by the Authentication Matrix test suite.

Since the fixed-configuration Access Point will not be managed by the AP-Control server, it does not have to be one of those from the list above.

Warning: selecting the –v option sends the program’s diagnostic output to the console window. If that is a normal Windows command console with the “Quick Edit Mode” option enabled, any select in the window will freeze the output. If APControl is writing messages to the console, the program will stop until the select is released. In the mean-time all the program’s clients will see the server become unresponsive and produce “Unable to configure Access Point” error messages.

Negative tests should not work. For example test number 8000 tries to connect with an Access Point configured to expect a WPA2-PSK connection using clear-text (no encryption). This is an invalid connection mode so it should never succeed.